SUPPLEMENTAL TABLES

Table S1. Microthermometry parameters, trapping pressure, time, and depth of the fluid inclusions, and corresponding lithostatic pressure of Well DY1 and DY3.

| Well name | Sample No. | Host mineral | FIAs No. | Aqueous inclusions (AIs) |  |  |  |  | Methane inclusion (GIs) |  |  |  |  | Trapping Time (Ma) | Trapping depth (m) | Plitho (MPa) | Pt/Plitho |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number | Th (℃) | Tm (℃) | Salinity (wt%) |  | Number | νm | ν1 | ρ (g/cm3) | Pt (MPa) |  |  |  |  |
| DY1 | DY1-M-1A | Cal-1 | F1 | 2 | 222.1 | -5.1 | 8.0 |  | 3 | 2909.9624 | 2910.9290 | 0.270 | 119.2 | 112.9 | 6637.7 | 165.9 | 0.72 |
|  |  |  |  |  | 223.2 | -5.0 | 7.9 |  |  |  |  |  | 119.6 | 111.6 | 6695.1 | 167.4 | 0.71 |
|  |  | Cal-1 | F2 | 2 | 223.8 | -4.8 | 7.6 |  | 2 | 2909.9624 | 2910.9290 | 0.270 | 119.8 | 111.0 | 6727.1 | 168.2 | 0.71 |
|  |  |  |  |  | 224.1 | -4.8 | 7.6 |  |  |  |  |  | 119.9 | 110.6 | 6743.0 | 168.6 | 0.71 |
|  |  | Cal-1 | F3 | 2 | 222.6 | -5.0 | 7.9 |  | 2 | 2909.9624 | 2910.9290 | 0.270 | 119.4 | 112.3 | 6663.2 | 166.6 | 0.72 |
|  |  |  |  |  | 223.0 | -5.0 | 7.9 |  |  |  |  |  | 119.5 | 111.9 | 6685.6 | 167.1 | 0.72 |
|  |  | Cal-2 | S3 | 2 | 203.8 | -4.5 | 7.2 |  | 2 | 2911.0066 | 2911.9705 | 0.221 | 75.7 | 98.1 | 6166.2 | 154.2 | 0.49 |
|  |  |  |  |  | 203.5 | -4.8 | 7.6 |  |  |  |  |  | 75.6 | 98.0 | 6158.6 | 154.0 | 0.49 |
|  | DY1-M-1B | Cal-1 | F1 | 5 | 223.9 |  |  |  | 3 | 2909.9624 | 2910.9290 | 0.270 | 119.8 | 110.9 | 6733.4 | 168.3 | 0.71 |
|  |  |  |  |  | 223.4 | -5.0 | 7.9 |  |  |  |  |  | 119.7 | 111.4 | 6698.3 | 167.5 | 0.71 |
|  |  |  |  |  | 218.5 | -5.2 | 8.1 |  |  |  |  |  | 118.0 | 116.9 | 6449.4 | 161.2 | 0.73 |
|  |  |  |  |  | 218.5 | -5.2 | 8.1 |  |  |  |  |  | 118.0 | 116.9 | 6449.4 | 161.2 | 0.73 |
|  |  |  |  |  | 218.8 | -5.2 | 8.1 |  |  |  |  |  | 118.1 | 116.6 | 6465.3 | 161.6 | 0.73 |
|  |  | Cal-1 | F2 | 2 | 224.0 | -5.0 | 7.9 |  | 2 | 2909.9624 | 2910.9290 | 0.270 | 119.9 | 110.7 | 6736.6 | 168.4 | 0.71 |
|  |  |  |  |  | 219.7 | -4.6 | 7.3 |  |  |  |  |  | 118.4 | 115.6 | 6513.2 | 162.8 | 0.73 |
|  |  | Cal-1 | F3 | 3 | 223.3 | -6.0 | 9.2 |  | 4 | 2909.9624 | 2910.9290 | 0.270 | 119.6 | 111.5 | 6698.3 | 167.5 | 0.71 |
|  |  |  |  |  | 221.4 | -5.2 | 8.1 |  |  |  |  |  | 119.0 | 113.6 | 6602.5 | 165.1 | 0.72 |
|  |  |  |  |  | 224.9 | -5.4 | 8.4 |  |  |  |  |  | 120.2 | 109.7 | 6784.5 | 169.6 | 0.71 |
|  |  | Cal-2 | S1 | 5 | 202.1 | -5.0 | 7.9 |  | 4 | 2911.0066 | 2911.9705 | 0.221 | 75.3 | 97.5 | 6120.9 | 153.0 | 0.49 |
|  |  |  |  |  | 198.8 | -4.8 | 7.6 |  |  |  |  |  | 74.5 | 96.3 | 6031.1 | 150.8 | 0.49 |
|  |  |  |  |  | 199.7 | -4.8 | 7.6 |  |  |  |  |  | 74.7 | 96.6 | 6055.9 | 151.4 | 0.49 |
|  |  |  |  |  | 202.1 | -3.0 | 5.0 |  |  |  |  |  | 75.3 | 97.5 | 6120.9 | 153.0 | 0.49 |
|  |  |  |  |  | 202.5 | -4.3 | 6.9 |  |  |  |  |  | 75.4 | 97.7 | 6131.6 | 153.3 | 0.49 |
|  |  | Cal-2 | S2 | 4 | 202.4 |  |  |  | 3 | 2911.0066 | 2911.9705 | 0.221 | 75.3 | 97.6 | 6128.4 | 153.2 | 0.49 |
|  |  |  |  |  | 201.8 | -4.6 | 7.3 |  |  |  |  |  | 75.2 | 97.4 | 6113.3 | 152.8 | 0.49 |
|  |  |  |  |  | 199.4 | -5.0 | 7.9 |  |  |  |  |  | 74.6 | 96.5 | 6047.2 | 151.2 | 0.49 |
|  |  |  |  |  | 199.4 |  |  |  |  |  |  |  | 74.6 | 96.5 | 6047.2 | 151.2 | 0.49 |
|  |  | Cal-2 | S3 | 4 | 203.5 |  |  |  | 4 | 2911.0066 | 2911.9705 | 0.221 | 75.6 | 98.0 | 6158.6 | 154.0 | 0.49 |
|  |  |  |  |  | 198.2 | -4.0 | 6.5 |  |  |  |  |  | 74.4 | 96.0 | 6013.7 | 150.3 | 0.49 |
|  |  |  |  |  | 199.7 | -6.0 | 9.2 |  |  |  |  |  | 74.7 | 96.6 | 6055.9 | 151.4 | 0.49 |
|  |  |  |  |  | 202.6 | -5.5 | 8.6 |  |  |  |  |  | 75.4 | 97.7 | 6133.5 | 153.3 | 0.49 |
| DY3 | DY3-M-2 | Cal-2 | S1 | 2 | 197.5 | -3.2 | 5.3 |  | 2 | 2908.9185 | 2911.9705 | 0.221 | 74.2 | 92.5 | 5985.2 | 149.6 | 0.50 |
|  |  |  |  |  | 195.8 |  |  |  |  |  |  |  | 73.8 | 91.8 | 5937.8 | 148.4 | 0.50 |
|  | DY3-M-3 | Cal-1 | F1 | 6 | 219.7 | -3.8 | 6.2 |  | 4 | 2907.1782 | 2910.9289 | 0.270 | 118.4 | 115.8 | 6463.5 | 161.6 | 0.73 |
|  |  |  |  |  | 221.6 |  |  |  |  |  |  |  | 119.0 | 113.6 | 6558.9 | 164.0 | 0.73 |
|  |  |  |  |  | 223.4 | -4.9 | 7.7 |  |  |  |  |  | 119.7 | 111.6 | 6649.3 | 166.2 | 0.72 |
|  |  |  |  |  | 222.8 | -5.0 | 7.9 |  |  |  |  |  | 119.5 | 112.3 | 6619.5 | 165.5 | 0.72 |
|  |  |  |  |  | 218.6 | -4.9 | 7.7 |  |  |  |  |  | 118.0 | 117.1 | 6407.2 | 160.2 | 0.74 |
|  |  |  |  |  | 222.4 | -5.1 | 8.0 |  |  |  |  |  | 119.3 | 112.7 | 6599.6 | 165.0 | 0.72 |
|  |  | Cal-1 | F2 | 5 | 223.7 | -5.0 | 7.9 |  | 3 | 2906.4822 | 2910.9290 | 0.270 | 119.8 | 111.3 | 6664.1 | 166.6 | 0.72 |
|  |  |  |  |  | 224.6 | -5.0 | 7.9 |  |  |  |  |  | 120.1 | 110.3 | 6708.7 | 167.7 | 0.72 |
|  |  |  |  |  | 225.3 | -4.8 | 7.6 |  |  |  |  |  | 120.3 | 109.5 | 6743.5 | 168.6 | 0.71 |
|  |  |  |  |  | 221.4 |  |  |  |  |  |  |  | 119.0 | 113.9 | 6549.1 | 163.7 | 0.73 |
|  |  |  |  |  | 223.3 | -5.0 | 7.9 |  |  |  |  |  | 119.6 | 111.7 | 6644.2 | 166.1 | 0.72 |
|  |  | Cal-1 | F3 | 5 | 218.9 | -4.0 | 6.5 |  | 4 | 2906.4822 | 2910.9290 | 0.270 | 118.1 | 116.8 | 6422.8 | 160.6 | 0.74 |
|  |  |  |  |  | 221.4 | -4.8 | 7.6 |  |  |  |  |  | 119.0 | 113.9 | 6549.1 | 163.7 | 0.73 |
|  |  |  |  |  | 223.1 |  |  |  |  |  |  |  | 119.6 | 112.0 | 6634.0 | 165.9 | 0.72 |
|  |  |  |  |  | 221.9 | -4.7 | 7.5 |  |  |  |  |  | 119.1 | 113.3 | 6574.6 | 164.4 | 0.72 |
|  |  |  |  |  | 222.6 | -5.1 | 8.0 |  |  |  |  |  | 119.4 | 112.5 | 6609.0 | 165.2 | 0.72 |
|  |  | Cal-2 | S1 | 5 | 198.3 | -4.8 | 7.6 |  | 4 | 2908.2224 | 2911.9705 | 0.221 | 74.4 | 92.9 | 6007.2 | 150.2 | 0.50 |
|  |  |  |  |  | 197.6 |  |  |  |  |  |  |  | 74.2 | 92.6 | 5987.9 | 149.7 | 0.50 |
|  |  |  |  |  | 197.9 | -4.9 | 7.7 |  |  |  |  |  | 74.3 | 92.7 | 5996.0 | 149.9 | 0.50 |
|  |  |  |  |  | 195.4 | -5.0 | 7.9 |  |  |  |  |  | 73.7 | 91.7 | 5926.7 | 148.2 | 0.50 |
|  |  |  |  |  | 196.1 | -5.0 | 7.9 |  |  |  |  |  | 73.9 | 92.0 | 5945.8 | 148.6 | 0.50 |
|  |  | Cal-2 | S2 | 6 | 201.5 | -5.0 | 7.9 |  | 5 | 2908.9185 | 2911.9705 | 0.221 | 75.1 | 94.1 | 6094.3 | 152.4 | 0.49 |
|  |  |  |  |  | 202.0 | -5.1 | 8.0 |  |  |  |  |  | 75.3 | 94.3 | 6107.6 | 152.7 | 0.49 |
|  |  |  |  |  | 198.6 | -5.2 | 8.1 |  |  |  |  |  | 74.4 | 93.0 | 6015.3 | 150.4 | 0.50 |
|  |  |  |  |  | 199.4 | -4.7 | 7.5 |  |  |  |  |  | 74.6 | 93.3 | 6037.4 | 150.9 | 0.49 |
|  |  |  |  |  | 197.3 | -5.0 | 7.9 |  |  |  |  |  | 74.1 | 92.5 | 5979.8 | 149.5 | 0.50 |
|  |  |  |  |  | 196.9 |  |  |  |  |  |  |  | 74.0 | 92.3 | 5968.5 | 149.2 | 0.50 |
|  | DY3-M-4 | Qtz-1 | F1 | 6 | 228.1 |  |  |  | 4 | 2907.1782 | 2910.9289 | 0.270 | 121.3 | 106.4 | 6863.3 | 171.6 | 0.71 |
|  |  |  |  |  | 224.2 |  |  |  |  |  |  |  | 119.9 | 110.7 | 6690.3 | 167.3 | 0.72 |
|  |  |  |  |  | 223.0 |  |  |  |  |  |  |  | 119.5 | 112.1 | 6634.0 | 165.9 | 0.72 |
|  |  |  |  |  | 225.7 |  |  |  |  |  |  |  | 120.4 | 109.0 | 6763.1 | 169.1 | 0.71 |
|  |  |  |  |  | 227.6 |  |  |  |  |  |  |  | 121.1 | 106.9 | 6807.2 | 170.2 | 0.71 |
|  |  |  |  |  | 226.0 |  |  |  |  |  |  |  | 120.5 | 108.7 | 6778.0 | 169.4 | 0.71 |
|  |  | Qtz-1 | F2 | 2 | 228.1 | -4.4 | 7.0 |  | 5 | 2906.8303 | 2910.9289 | 0.270 | 121.3 | 106.4 | 6896.9 | 172.4 | 0.70 |
|  |  |  |  |  | 224.5 | -4.8 | 7.6 |  |  |  |  |  | 120.0 | 110.4 | 6704.0 | 167.6 | 0.72 |
|  |  | Qtz-1 | F3 | 3 | 227.6 |  |  |  | 5 | 2906.4822 | 2910.9290 | 0.270 | 121.1 | 106.9 | 6807.2 | 170.2 | 0.71 |
|  |  |  |  |  | 226.4 |  |  |  |  |  |  |  | 120.7 | 108.3 | 6796.7 | 169.9 | 0.71 |
|  |  |  |  |  | 223.4 |  |  |  |  |  |  |  | 119.7 | 111.6 | 6649.6 | 166.2 | 0.72 |
|  |  | Qtz-2 | S1 | 1 | 202.4 |  |  |  | 2 | 2908.2224 | 2911.9705 | 0.221 | 75.3 | 94.5 | 6118.6 | 153.0 | 0.49 |
|  |  | Qtz-2 | S2 | 1 | 202.6 |  |  |  | 2 | 2908.2224 | 2911.9705 | 0.221 | 75.4 | 94.6 | 6124.1 | 153.1 | 0.49 |
|  |  | Qtz-2 | S3 | 2 | 207.3 | -4.7 | 7.5 |  | 3 | 2907.5264 | 2911.9705 | 0.221 | 76.5 | 96.4 | 6248.8 | 156.2 | 0.49 |
|  |  |  |  |  | 203.9 |  |  |  |  |  |  |  | 75.7 | 95.1 | 6159.2 | 154.0 | 0.49 |
|  |  | Qtz-2 | S4 | 3 | 194.9 | -4.3 | 6.9 |  | 3 | 2907.5264 | 2911.9705 | 0.221 | 73.6 | 91.5 | 5913.0 | 147.8 | 0.50 |
|  |  |  |  |  | 195.1 |  |  |  |  |  |  |  | 73.6 | 91.6 | 5918.4 | 148.0 | 0.50 |
|  |  |  |  |  | 197.0 |  |  |  |  |  |  |  | 74.1 | 92.3 | 5972.0 | 149.3 | 0.50 |

Abbreviations: Cal-1 = first-generation calcite; Cal-2 = second-generation calcite; Qtz-1= first-generation quartz; Qtz-2 = second-generation quartz; FIAs = fluid inclusion assemblage; Th = homogenization temperature; Tm = final ice-melting temperature; νm = Raman CH4 peak shift; ν1 = real Raman CH4 peak position; ρ = methane density; Pt = trapping pressure; Plitho = (Trapping depth/1000) × 25 MPa/km, lithostatic pressure.

Table S2. Microthermometry parameters, trapping pressure, time, and depth of the fluid inclusions, and corresponding lithostatic pressure of Well DY4, DY5, and DY2.

| Well name | Sample No. | Host mineral | FIAs No. | Aqueous inclusions (AIs) |  |  |  |  | Methane inclusion (GIs) |  |  |  |  | Trapping Time (Ma) | Trapping depth (m) | Plitho (MPa) | Pt/Plitho |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number | Th (℃) | Tm (℃) | Salinity (wt%) |  | Number | νm | ν1 | ρ (g/cm3) | Pt (MPa) |  |  |  |  |
| DY4 | DY4-M-4 | Cal-1 | F1 | 2 | 235.2 |  |  |  | 2 | 2909.6145 | 2910.7315 | 0.280 | 133.9 | 113.2 | 7022.0 | 175.6 | 0.76 |
|  |  |  |  |  | 233.9 |  |  |  |  |  |  |  | 133.4 | 114.6 | 6956.2 | 173.9 | 0.77 |
|  |  | Cal-2 | S1 | 3 | 186.7 |  |  |  | 5 | 2910.3105 | 2911.4271 | 0.246 | 87.9 | 71.8 | 5791.5 | 144.8 | 0.61 |
|  |  |  |  |  | 186.7 |  |  |  |  |  |  |  | 87.9 | 71.8 | 5791.5 | 144.8 | 0.61 |
|  |  |  |  |  | 188.1 |  |  |  |  |  |  |  | 88.3 | 74.3 | 5826.6 | 145.7 | 0.61 |
|  | DY4-M-5 | Cal-1 | F1 | 3 | 239.0 |  |  |  | 4 | 2909.2664 | 2910.7315 | 0.280 | 135.3 | 109.1 | 7215.2 | 180.4 | 0.75 |
|  |  |  |  |  | 239.5 | -8.6 | 12.4 |  |  |  |  |  | 135.4 | 108.6 | 7240.2 | 181.0 | 0.75 |
|  |  |  |  |  | 237.0 | -8.7 | 12.5 |  |  |  |  |  | 134.5 | 111.2 | 7113.6 | 177.8 | 0.76 |
|  |  | Cal-1 | F2 | 3 | 232.2 |  |  |  | 4 | 2909.2664 | 2910.7315 | 0.280 | 132.8 | 116.4 | 6870.2 | 171.8 | 0.77 |
|  |  |  |  |  | 231.7 |  |  |  |  |  |  |  | 132.6 | 116.9 | 6843.5 | 171.1 | 0.77 |
|  |  |  |  |  | 235.3 |  |  |  |  |  |  |  | 133.9 | 113.1 | 7027.2 | 175.7 | 0.76 |
|  |  | Cal-1 | F3 | 3 | 233.7 | -8.6 | 12.4 |  | 3 | 2909.2664 | 2910.7315 | 0.280 | 133.3 | 114.8 | 6946.2 | 173.7 | 0.77 |
|  |  |  |  |  | 237.8 | -8.4 | 12.2 |  |  |  |  |  | 134.8 | 110.4 | 7154.1 | 178.9 | 0.75 |
|  |  |  |  |  | 234.5 | -8.5 | 12.3 |  |  |  |  |  | 133.6 | 113.9 | 6985.7 | 174.6 | 0.77 |
|  |  | Cal-2 | S1 | 3 | 185.4 | -8.0 | 11.7 |  | 3 | 2909.9624 | 2911.4271 | 0.246 | 87.5 | 68.5 | 5763.7 | 144.1 | 0.61 |
|  |  |  |  |  | 187.5 | -7.3 | 10.9 |  |  |  |  |  | 88.2 | 73.9 | 5808.8 | 145.2 | 0.61 |
|  |  |  |  |  | 189.0 |  |  |  |  |  |  |  | 88.6 | 74.7 | 5853.0 | 146.3 | 0.61 |
|  |  | Cal-2 | S2 | 3 | 187.4 |  |  |  | 2 | 2909.9624 | 2911.4271 | 0.246 | 88.1 | 73.6 | 5806.5 | 145.2 | 0.61 |
|  |  |  |  |  | 185.2 |  |  |  |  |  |  |  | 87.5 | 68.0 | 5759.5 | 144.0 | 0.61 |
|  |  |  |  |  | 187.1 |  |  |  |  |  |  |  | 88.0 | 72.8 | 5800.4 | 145.0 | 0.61 |
|  |  | Cal-2 | S3 | 5 | 184.8 | -7.4 | 11.0 |  | 4 | 2909.9624 | 2911.4271 | 0.246 | 87.4 | 66.7 | 5748.5 | 143.7 | 0.61 |
|  |  |  |  |  | 187.2 |  |  |  |  |  |  |  | 88.1 | 73.1 | 5802.5 | 145.1 | 0.61 |
|  |  |  |  |  | 185.4 |  |  |  |  |  |  |  | 87.5 | 68.5 | 5763.7 | 144.1 | 0.61 |
|  |  |  |  |  | 185.4 |  |  |  |  |  |  |  | 87.5 | 68.5 | 5763.7 | 144.1 | 0.61 |
|  |  |  |  |  | 183.2 |  |  |  |  |  |  |  | 86.9 | 61.3 | 5696.0 | 142.4 | 0.61 |
|  | DY4-M-6 | Cal-1 | F1 | 4 | 238.8 | -7.6 | 11.2 |  | 5 | 2909.2664 | 2910.7315 | 0.280 | 135.2 | 109.3 | 7204.9 | 180.1 | 0.75 |
|  |  |  |  |  | 239.0 | -7.5 | 11.1 |  |  |  |  |  | 135.3 | 109.1 | 7215.2 | 180.4 | 0.75 |
|  |  |  |  |  | 239.7 | -7.6 | 11.2 |  |  |  |  |  | 135.5 | 108.3 | 7250.1 | 181.3 | 0.75 |
|  |  |  |  |  | 241.5 | -7.5 | 11.1 |  |  |  |  |  | 136.2 | 106.4 | 7340.7 | 183.5 | 0.74 |
|  |  | Cal-1 | F2 | 2 | 243.4 | -7.7 | 11.3 |  | 2 | 2909.2664 | 2910.7315 | 0.280 | 136.9 | 104.4 | 7436.7 | 185.9 | 0.74 |
|  |  |  |  |  | 239.1 | -7.5 | 11.1 |  |  |  |  |  | 135.3 | 109.0 | 7220.4 | 180.5 | 0.75 |
|  |  | Cal-1 | F3 | 4 | 229.4 | -7.2 | 10.7 |  | 3 | 2909.2664 | 2910.7315 | 0.280 | 131.7 | 119.3 | 6731.6 | 168.3 | 0.78 |
|  |  |  |  |  | 233.0 | -7.4 | 11.0 |  |  |  |  |  | 133.1 | 115.5 | 6911.1 | 172.8 | 0.77 |
|  |  |  |  |  | 230.8 | -7.6 | 11.2 |  |  |  |  |  | 132.3 | 117.9 | 6800.3 | 170.0 | 0.78 |
|  |  |  |  |  | 233.8 | -7.4 | 11.0 |  |  |  |  |  | 133.4 | 114.7 | 6951.0 | 173.8 | 0.77 |
|  |  | Cal-1 | F4 | 3 | 241.5 | -7.4 | 11.0 |  | 4 | 2909.2664 | 2910.7315 | 0.280 | 136.2 | 106.4 | 7340.7 | 183.5 | 0.74 |
|  |  |  |  |  | 238.3 |  |  |  |  |  |  |  | 135.0 | 109.9 | 7179.1 | 179.5 | 0.75 |
|  |  |  |  |  | 236.6 | -7.2 | 10.7 |  |  |  |  |  | 134.4 | 111.7 | 7093.3 | 177.3 | 0.76 |
|  |  | Cal-2 | S1 | 4 | 185.4 | -7.8 | 11.5 |  | 4 | 2909.9624 | 2911.4271 | 0.246 | 87.5 | 68.5 | 5763.7 | 144.1 | 0.61 |
|  |  |  |  |  | 184.1 | -6.8 | 10.2 |  |  |  |  |  | 87.2 | 64.4 | 5724.8 | 143.1 | 0.61 |
|  |  |  |  |  | 182.4 | -7.5 | 11.1 |  |  |  |  |  | 86.7 | 58.4 | 5668.8 | 141.7 | 0.61 |
|  |  |  |  |  | 187.1 | -7.4 | 11.0 |  |  |  |  |  | 88.0 | 72.8 | 5800.4 | 145.0 | 0.61 |
|  |  | Cal-2 | S2 | 2 | 187.4 | -5.9 | 9.1 |  | 3 | 2909.9624 | 2911.4271 | 0.246 | 88.1 | 73.6 | 5806.5 | 145.2 | 0.61 |
|  |  |  |  |  | 182.3 | -5.6 | 8.7 |  |  |  |  |  | 86.6 | 58.1 | 5665.0 | 141.6 | 0.61 |
|  |  | Cal-2 | S3 | 2 | 188.9 | -5 | 7.9 |  | 3 | 2909.9624 | 2911.4271 | 0.246 | 88.6 | 74.7 | 5850.6 | 146.3 | 0.61 |
|  |  |  |  |  | 193.4 | -8 | 11.7 |  |  |  |  |  | 89.9 | 77.1 | 5988.7 | 149.7 | 0.60 |
|  |  | Cal-2 | S4 | 4 | 187.6 | -7.7 | 11.3 |  | 4 | 2909.9624 | 2911.4271 | 0.246 | 88.2 | 74.0 | 5811.4 | 145.3 | 0.61 |
|  |  |  |  |  | 187.2 | -7.6 | 11.2 |  |  |  |  |  | 88.1 | 73.1 | 5802.5 | 145.1 | 0.61 |
|  |  |  |  |  | 185.8 | -6.8 | 10.2 |  |  |  |  |  | 87.7 | 69.5 | 5772.4 | 144.3 | 0.61 |
|  |  |  |  |  | 183.7 | -7.6 | 11.2 |  |  |  |  |  | 87.0 | 63.0 | 5712.2 | 142.8 | 0.61 |
| DY5 | DY5-M-1 | Cal-1 | F1 | 5 | 239.2 |  |  |  | 2 | 2906.1343 | 2910.7315 | 0.280 | 135.3 | 108.4 | 7232.7 | 180.8 | 0.75 |
|  |  |  |  |  | 235.4 |  |  |  |  |  |  |  | 133.9 | 112.3 | 7053.6 | 176.3 | 0.76 |
|  |  |  |  |  | 234.1 |  |  |  |  |  |  |  | 133.5 | 113.6 | 6991.8 | 174.8 | 0.76 |
|  |  |  |  |  | 235.5 | -3.8 | 6.2 |  |  |  |  |  | 134.0 | 112.2 | 7058.1 | 176.5 | 0.76 |
|  |  |  |  |  | 236.4 |  |  |  |  |  |  |  | 134.3 | 111.3 | 7100.9 | 177.5 | 0.76 |
|  |  | Cal-2 | S1 | 2 | 182.2 | -3.6 | 5.9 |  | 2 | 2909.2664 | 2911.4272 | 0.246 | 86.6 | 51.3 | 5691.3 | 142.3 | 0.61 |
|  |  |  |  |  | 182.5 | -3.5 | 5.7 |  |  |  |  |  | 86.7 | 52.4 | 5701.7 | 142.5 | 0.61 |
|  |  | Cal-2 | S2 | 2 | 183.6 | -3.4 | 5.6 |  | 2 | 2909.2664 | 2911.4272 | 0.246 | 87.0 | 56.5 | 5739.3 | 143.5 | 0.61 |
|  |  |  |  |  | 184.7 | -3.6 | 5.9 |  |  |  |  |  | 87.3 | 60.4 | 5776.1 | 144.4 | 0.60 |
|  |  | Qtz-1 | F1 | 5 | 239.7 | -3.6 | 5.9 |  | 4 | 2906.1343 | 2910.7315 | 0.280 | 135.5 | 107.9 | 7256.0 | 181.4 | 0.75 |
|  |  |  |  |  | 238.1 | -3.8 | 6.2 |  |  |  |  |  | 134.9 | 109.5 | 7181.0 | 179.5 | 0.75 |
|  |  |  |  |  | 239.0 | -3.6 | 5.9 |  |  |  |  |  | 135.3 | 108.6 | 7223.4 | 180.6 | 0.75 |
|  |  |  |  |  | 239.3 | -3.3 | 5.4 |  |  |  |  |  | 135.4 | 108.3 | 7237.2 | 180.9 | 0.75 |
|  |  |  |  |  | 234.7 | -3.6 | 5.9 |  |  |  |  |  | 133.7 | 113.0 | 7020.4 | 175.5 | 0.76 |
|  |  | Qtz-2 | S1 | 1 | 183.2 |  |  |  | 2 | 2909.2664 | 2911.4272 | 0.246 | 86.9 | 55.0 | 5725.6 | 143.1 | 0.61 |
|  | DY5-M-2 | Qtz-1 | F1 | 4 | 238.8 |  |  |  | 5 | 2905.7864 | 2910.7316 | 0.280 | 135.2 | 108.8 | 7213.6 | 180.3 | 0.75 |
|  |  |  |  |  | 236.0 |  |  |  |  |  |  |  | 134.2 | 111.7 | 7081.9 | 177.0 | 0.76 |
|  |  |  |  |  | 237.5 |  |  |  |  |  |  |  | 134.7 | 110.1 | 7152.7 | 178.8 | 0.75 |
|  |  |  |  |  | 236.5 | -3.6 | 5.9 |  |  |  |  |  | 134.3 | 111.2 | 7106.1 | 177.7 | 0.76 |
|  |  | Qtz-1 | F2 | 2 | 239.6 | -3.5 | 5.7 |  | 3 | 2906.4822 | 2910.7314 | 0.280 | 135.4 | 108.0 | 7251.1 | 181.3 | 0.75 |
|  |  |  |  |  | 242.1 | -4.0 | 6.5 |  |  |  |  |  | 136.4 | 105.5 | 7368.1 | 184.2 | 0.74 |
|  |  | Qtz-1 | F3 | 3 | 237.2 |  |  |  | 3 | 2908.5703 | 2910.7315 | 0.280 | 134.6 | 110.5 | 7138.8 | 178.5 | 0.75 |
|  |  |  |  |  | 233.6 | -3.7 | 6.0 |  |  |  |  |  | 133.3 | 114.1 | 6967.8 | 174.2 | 0.77 |
|  |  |  |  |  | 238.9 | -5.0 | 7.9 |  |  |  |  |  | 135.2 | 108.7 | 7218.4 | 180.5 | 0.75 |
|  |  | Qtz-1 | F4 | 3 | 234.7 | -3.1 | 5.1 |  | 4 | 2907.8743 | 2910.7315 | 0.280 | 133.7 | 113.0 | 7020.4 | 175.5 | 0.76 |
|  |  |  |  |  | 239.8 |  |  |  |  |  |  |  | 135.5 | 107.8 | 7260.5 | 181.5 | 0.75 |
|  |  |  |  |  | 236.4 | -4.1 | 6.6 |  |  |  |  |  | 134.3 | 111.3 | 7100.7 | 177.5 | 0.76 |
|  |  | Qtz-2 | S1 | 3 | 185.7 |  |  |  | 4 | 2907.5264 | 2911.4272 | 0.246 | 87.6 | 64.0 | 5809.5 | 145.2 | 0.60 |
|  |  |  |  |  | 184.7 |  |  |  |  |  |  |  | 87.3 | 60.4 | 5776.1 | 144.4 | 0.60 |
|  |  |  |  |  | 188.3 |  |  |  |  |  |  |  | 88.4 | 71.8 | 5873.2 | 146.8 | 0.60 |
|  |  | Qtz-2 | S2 | 2 | 184.8 |  |  |  | 2 | 2908.2224 | 2911.4271 | 0.246 | 87.4 | 60.8 | 5779.4 | 144.5 | 0.60 |
|  |  |  |  |  | 183.1 |  |  |  |  |  |  |  | 86.9 | 54.6 | 5722.2 | 143.1 | 0.61 |
|  | DY5-M-3 | Cal-1 | F1 | 7 | 235.0 | -5.0 | 7.9 |  | 2 | 2906.1343 | 2910.7315 | 0.280 | 133.8 | 112.7 | 7034.3 | 175.9 | 0.76 |
|  |  |  |  |  | 237.0 | -3.8 | 6.2 |  |  |  |  |  | 134.5 | 110.6 | 7129.4 | 178.2 | 0.75 |
|  |  |  |  |  | 235.6 | -4.0 | 6.5 |  |  |  |  |  | 134.0 | 112.1 | 7063.0 | 176.6 | 0.76 |
|  |  |  |  |  | 236.5 | -5.0 | 7.9 |  |  |  |  |  | 134.3 | 111.2 | 7105.6 | 177.6 | 0.76 |
|  |  |  |  |  | 233.4 | -3.8 | 6.2 |  |  |  |  |  | 133.2 | 114.3 | 6958.4 | 174.0 | 0.77 |
|  |  |  |  |  | 237.3 |  |  |  |  |  |  |  | 134.6 | 110.4 | 7143.3 | 178.6 | 0.75 |
|  |  |  |  |  | 237.8 |  |  |  |  |  |  |  | 134.8 | 109.8 | 7166.8 | 179.2 | 0.75 |
|  |  | Cal-1 | F2 | 4 | 229.4 | -3.7 | 6.0 |  | 4 | 2906.1343 | 2910.7315 | 0.280 | 131.7 | 118.4 | 6765.3 | 169.1 | 0.78 |
|  |  |  |  |  | 231.5 | -3.6 | 5.9 |  |  |  |  |  | 132.5 | 116.3 | 6865.0 | 171.6 | 0.77 |
|  |  |  |  |  | 234.3 | -4.5 | 7.2 |  |  |  |  |  | 133.5 | 113.4 | 7001.5 | 175.0 | 0.76 |
|  |  |  |  |  | 233.8 | -4.2 | 6.7 |  |  |  |  |  | 133.4 | 113.9 | 6977.4 | 174.4 | 0.76 |
|  |  | Cal-1 | F3 | 5 | 232.8 |  |  |  | 4 | 2906.1343 | 2910.7315 | 0.280 | 133.0 | 114.9 | 6929.3 | 173.2 | 0.77 |
|  |  |  |  |  | 237.6 | -2.8 | 4.7 |  |  |  |  |  | 134.7 | 110.0 | 7157.4 | 178.9 | 0.75 |
|  |  |  |  |  | 232.2 | -5.0 | 7.9 |  |  |  |  |  | 132.8 | 115.6 | 6900.0 | 172.5 | 0.77 |
|  |  |  |  |  | 234.0 |  |  |  |  |  |  |  | 133.4 | 113.7 | 6986.7 | 174.7 | 0.76 |
|  |  |  |  |  | 235.0 | -3.5 | 5.7 |  |  |  |  |  | 133.8 | 112.7 | 7034.3 | 175.9 | 0.76 |
|  |  | Cal-2 | S1 | 4 | 184.3 | -4.1 | 6.6 |  | 3 | 2907.5264 | 2911.4272 | 0.246 | 87.2 | 59.0 | 5762.9 | 144.1 | 0.61 |
|  |  |  |  |  | 185.1 | -3.8 | 6.2 |  |  |  |  |  | 87.5 | 61.9 | 5789.2 | 144.7 | 0.60 |
|  |  |  |  |  | 183.6 | -3.6 | 5.9 |  |  |  |  |  | 87.0 | 56.5 | 5739.3 | 143.5 | 0.61 |
|  |  |  |  |  | 183.2 | -3.6 | 5.9 |  |  |  |  |  | 86.9 | 55.0 | 5725.6 | 143.1 | 0.61 |
|  |  | Cal-2 | S2 | 5 | 182.7 | -3.7 | 6.0 |  | 4 | 2909.2664 | 2911.4272 | 0.246 | 86.8 | 53.2 | 5708.3 | 142.7 | 0.61 |
|  |  |  |  |  | 182.5 |  |  |  |  |  |  |  | 86.7 | 52.4 | 5701.7 | 142.5 | 0.61 |
|  |  |  |  |  | 183.6 | -4.0 | 6.5 |  |  |  |  |  | 87.0 | 56.5 | 5739.3 | 143.5 | 0.61 |
|  |  |  |  |  | 184.1 | -3.6 | 5.9 |  |  |  |  |  | 87.2 | 58.3 | 5755.9 | 143.9 | 0.61 |
|  |  |  |  |  | 183.9 | -3.8 | 6.2 |  |  |  |  |  | 87.1 | 57.6 | 5749.3 | 143.7 | 0.61 |
|  |  | Qtz-1 | F1 | 6 | 232.7 | -3.4 | 5.6 |  | 4 | 2906.1343 | 2910.7315 | 0.280 | 133.0 | 115.0 | 6924.7 | 173.1 | 0.77 |
|  |  |  |  |  | 233.7 | -3.3 | 5.4 |  |  |  |  |  | 133.3 | 114.0 | 6972.8 | 174.3 | 0.76 |
|  |  |  |  |  | 234.6 | -3.3 | 5.4 |  |  |  |  |  | 133.6 | 113.1 | 7015.5 | 175.4 | 0.76 |
|  |  |  |  |  | 233.3 | -2.0 | 3.4 |  |  |  |  |  | 133.2 | 114.4 | 6953.5 | 173.8 | 0.77 |
|  |  |  |  |  | 234.7 | -3.7 | 6.0 |  |  |  |  |  | 133.7 | 113.0 | 7020.4 | 175.5 | 0.76 |
|  |  |  |  |  | 234.0 | -3.6 | 5.9 |  |  |  |  |  | 133.4 | 113.7 | 6986.7 | 174.7 | 0.76 |
|  |  | Qtz-1 | F2 | 8 | 236.8 |  |  |  | 5 | 2906.1343 | 2910.7315 | 0.280 | 134.5 | 110.9 | 7119.5 | 178.0 | 0.76 |
|  |  |  |  |  | 232.3 | -3.6 | 5.9 |  |  |  |  |  | 132.8 | 115.4 | 6905.0 | 172.6 | 0.77 |
|  |  |  |  |  | 234.1 | -4.0 | 6.5 |  |  |  |  |  | 133.5 | 113.6 | 6991.9 | 174.8 | 0.76 |
|  |  |  |  |  | 236.3 | -3.7 | 6.0 |  |  |  |  |  | 134.3 | 111.4 | 7096.0 | 177.4 | 0.76 |
|  |  |  |  |  | 236.1 |  |  |  |  |  |  |  | 134.2 | 111.6 | 7086.8 | 177.2 | 0.76 |
|  |  |  |  |  | 237.4 | -4.0 | 6.5 |  |  |  |  |  | 134.7 | 110.3 | 7147.7 | 178.7 | 0.75 |
|  |  |  |  |  | 232.8 |  |  |  |  |  |  |  | 133.0 | 114.9 | 6929.4 | 173.2 | 0.77 |
|  |  |  |  |  | 236.9 | -3.7 | 6.0 |  |  |  |  |  | 134.5 | 110.8 | 7124.4 | 178.1 | 0.76 |
|  |  | Qtz-2 | S1 | 3 | 188.1 | -3.6 | 5.9 |  | 4 | 2906.8303 | 2911.4271 | 0.246 | 88.3 | 71.2 | 5868.5 | 146.7 | 0.60 |
|  |  |  |  |  | 186.4 | -3.7 | 6.0 |  |  |  |  |  | 87.8 | 66.2 | 5828.0 | 145.7 | 0.60 |
|  |  |  |  |  | 187.2 | -3.6 | 5.9 |  |  |  |  |  | 88.1 | 68.6 | 5847.1 | 146.2 | 0.60 |
|  |  | Qtz-2 | S2 | 4 | 185.4 | -3.5 | 5.7 |  | 4 | 2906.8303 | 2911.4271 | 0.246 | 87.5 | 62.9 | 5799.3 | 145.0 | 0.60 |
|  |  |  |  |  | 188.6 | -3.7 | 6.0 |  |  |  |  |  | 88.5 | 72.7 | 5880.3 | 147.0 | 0.60 |
|  |  |  |  |  | 184.9 |  |  |  |  |  |  |  | 87.4 | 61.1 | 5782.8 | 144.6 | 0.60 |
|  |  |  |  |  | 187.3 | -3.8 | 6.2 |  |  |  |  |  | 88.1 | 68.8 | 5849.5 | 146.2 | 0.60 |
|  |  | Qtz-2 | S3 | 3 | 187.7 | -3.6 | 5.9 |  | 3 | 2908.2224 | 2911.4271 | 0.246 | 88.2 | 70.0 | 5859.0 | 146.5 | 0.60 |
|  |  |  |  |  | 185.6 | -3.7 | 6.0 |  |  |  |  |  | 87.6 | 63.7 | 5806.1 | 145.2 | 0.60 |
|  |  |  |  |  | 184.4 | -3.6 | 5.9 |  |  |  |  |  | 87.3 | 59.3 | 5765.9 | 144.1 | 0.61 |
|  | DY5-M-7 | Cal-1 | F1 | 3 | 234.8 | -4.6 | 7.3 |  | 1 | 2906.8303 | 2910.7314 | 0.280 | 133.7 | 112.9 | 7025.1 | 175.6 | 0.76 |
|  |  |  |  |  | 232.5 | -4.2 | 6.7 |  |  |  |  |  | 132.9 | 115.2 | 6914.8 | 172.9 | 0.77 |
|  |  |  |  |  | 233.6 | -4.4 | 7.0 |  |  |  |  |  | 133.3 | 114.1 | 6967.8 | 174.2 | 0.77 |
|  |  | Cal-1 | F2 | 4 | 237.5 |  |  |  | 3 | 2906.8303 | 2910.7314 | 0.280 | 134.7 | 110.1 | 7152.4 | 178.8 | 0.75 |
|  |  |  |  |  | 235.1 |  |  |  |  |  |  |  | 133.8 | 112.6 | 7039.2 | 176.0 | 0.76 |
|  |  |  |  |  | 235.5 | -3.4 | 5.6 |  |  |  |  |  | 134.0 | 112.2 | 7058.1 | 176.5 | 0.76 |
|  |  |  |  |  | 236.7 | -4.2 | 6.7 |  |  |  |  |  | 133.4 | 111.0 | 7115.1 | 177.9 | 0.75 |
|  |  | Cal-2 | S1 | 6 | 185.7 |  |  |  | 3 | 2907.5264 | 2911.4272 | 0.246 | 87.6 | 64.0 | 5809.5 | 145.2 | 0.60 |
|  |  |  |  |  | 187.1 |  |  |  |  |  |  |  | 88.0 | 68.3 | 5844.6 | 146.1 | 0.60 |
|  |  |  |  |  | 186.7 |  |  |  |  |  |  |  | 87.9 | 67.1 | 5835.1 | 145.9 | 0.60 |
|  |  |  |  |  | 183.4 |  |  |  |  |  |  |  | 87.0 | 55.7 | 5732.3 | 143.3 | 0.61 |
|  |  |  |  |  | 185.8 |  |  |  |  |  |  |  | 87.7 | 64.4 | 5812.9 | 145.3 | 0.60 |
|  |  |  |  |  | 187.5 |  |  |  |  |  |  |  | 88.1 | 69.4 | 5854.2 | 146.4 | 0.60 |
|  |  | Cal-2 | S2 | 1 | 189.0 | -4.0 | 6.5 |  | 2 | 2907.5264 | 2911.4272 | 0.246 | 88.6 | 73.1 | 5891.4 | 147.3 | 0.60 |
|  | DY5-M-8 | Cal-1 | F1 | 3 | 232.0 | -4.3 | 6.9 |  | 4 | 2906.8303 | 2910.7314 | 0.280 | 132.7 | 115.8 | 6890.2 | 172.3 | 0.77 |
|  |  |  |  |  | 234.8 | -4.3 | 6.9 |  |  |  |  |  | 133.7 | 112.9 | 7025.1 | 175.6 | 0.76 |
|  |  |  |  |  | 233.1 | -4.2 | 6.7 |  |  |  |  |  | 133.1 | 114.6 | 6943.8 | 173.6 | 0.77 |
| DY2 | DY2-M-2 | Cal-1 | F1 | 3 | 236.8 | -6.2 | 9.5 |  | 3 | 2909.6145 | 2910.7315 | 0.280 | 134.5 | 115.8 | 7020.7 | 175.5 | 0.77 |
|  |  |  |  |  | 237.6 | -5.0 | 7.9 |  |  |  |  |  | 134.7 | 114.9 | 7059.2 | 176.5 | 0.76 |
|  |  |  |  |  | 239.4 |  |  |  |  |  |  |  | 135.4 | 112.8 | 7142.9 | 178.6 | 0.76 |
|  |  | Cal-2 | S1 | 3 | 184.3 |  |  |  | 2 | 2910.6584 | 2911.4271 | 0.246 | 87.2 | 32.2 | 5753.2 | 143.8 | 0.61 |
|  |  |  |  |  | 186.6 |  |  |  |  |  |  |  | 87.9 | 34.1 | 5834.9 | 145.9 | 0.60 |
|  |  |  |  |  | 189.8 | -7.6 | 11.2 |  |  |  |  |  | 88.8 | 38.5 | 5962.8 | 149.1 | 0.60 |
|  | DY2-M-3A | Cal-1 | F1 | 3 | 236.7 | -6.8 | 10.2 |  | 3 | 2909.6145 | 2910.7315 | 0.280 | 134.4 | 115.9 | 7015.9 | 175.4 | 0.77 |
|  |  |  |  |  | 235.9 | -6.8 | 10.2 |  |  |  |  |  | 134.1 | 116.9 | 6977.8 | 174.4 | 0.77 |
|  |  |  |  |  | 233.2 | -7.1 | 10.6 |  |  |  |  |  | 133.1 | 119.9 | 6856.5 | 171.4 | 0.78 |
|  |  | Cal-1 | F2 | 4 | 234.5 |  |  |  | 3 | 2909.6145 | 2910.7315 | 0.280 | 133.6 | 118.4 | 6916.5 | 172.9 | 0.77 |
|  |  |  |  |  | 239.5 |  |  |  |  |  |  |  | 135.4 | 112.6 | 7147.5 | 178.7 | 0.76 |
|  |  |  |  |  | 239.2 |  |  |  |  |  |  |  | 135.3 | 113.0 | 7133.6 | 178.3 | 0.76 |
|  |  |  |  |  | 236.7 |  |  |  |  |  |  |  | 134.4 | 115.9 | 7015.9 | 175.4 | 0.77 |
|  |  | Cal-1 | F3 | 2 | 241.5 | -8.0 | 11.7 |  | 2 | 2909.6145 | 2910.7315 | 0.280 | 136.2 | 110.3 | 7239.9 | 181.0 | 0.75 |
|  |  |  |  |  | 243.3 | -7.6 | 11.2 |  |  |  |  |  | 136.8 | 108.3 | 7323.0 | 183.1 | 0.75 |
|  |  | Cal-2 | S1 | 3 | 184.6 | -6.0 | 9.2 |  | 4 | 2909.9624 | 2911.4271 | 0.246 | 87.3 | 32.5 | 5763.5 | 144.1 | 0.61 |
|  |  |  |  |  | 187.3 | -6.2 | 9.5 |  |  |  |  |  | 88.1 | 34.7 | 5861.3 | 146.5 | 0.60 |
|  |  |  |  |  | 188.6 | -8.8 | 12.6 |  |  |  |  |  | 88.5 | 36.1 | 5915.6 | 147.9 | 0.60 |
|  |  | Cal-2 | S2 | 3 | 184.8 |  |  |  | 5 | 2909.9624 | 2911.4271 | 0.246 | 87.4 | 32.6 | 5770.8 | 144.3 | 0.61 |
|  |  |  |  |  | 186.6 |  |  |  |  |  |  |  | 87.9 | 34.1 | 5834.9 | 145.9 | 0.60 |
|  |  |  |  |  | 188.6 | -8.8 | 12.6 |  |  |  |  |  | 88.5 | 36.1 | 5915.6 | 147.9 | 0.60 |
|  |  | Cal-2 | S3 | 2 | 189.2 | -6.5 | 9.9 |  | 3 | 2909.9624 | 2911.4271 | 0.246 | 88.6 | 36.9 | 5942.2 | 148.6 | 0.60 |
|  |  |  |  |  | 186.0 | -6.8 | 10.2 |  |  |  |  |  | 87.7 | 33.6 | 5812.1 | 145.3 | 0.60 |
|  | DY2-M-3B | Cal-1 | F1 | 3 | 236.0 | -7.2 | 10.7 |  | 2 | 2909.2664 | 2910.7315 | 0.280 | 134.2 | 116.8 | 6981.9 | 174.5 | 0.77 |
|  |  |  |  |  | 234.2 | -7.4 | 11.0 |  |  |  |  |  | 133.5 | 118.7 | 6902.5 | 172.6 | 0.77 |
|  |  |  |  |  | 238.8 | -7.2 | 10.7 |  |  |  |  |  | 135.2 | 113.5 | 7115.0 | 177.9 | 0.76 |
|  |  | Cal-1 | F2 | 3 | 237.6 | -7.9 | 11.6 |  | 4 | 2909.2664 | 2910.7315 | 0.280 | 134.7 | 114.9 | 7059.2 | 176.5 | 0.76 |
|  |  |  |  |  | 242.5 |  |  |  |  |  |  |  | 136.5 | 109.2 | 7286.1 | 182.2 | 0.75 |
|  |  |  |  |  | 238.0 | -7.7 | 11.3 |  |  |  |  |  | 134.9 | 114.4 | 7078.0 | 177.0 | 0.76 |
|  |  | Cal-1 | F3 | 3 | 239.5 | -8.0 | 11.7 |  | 3 | 2909.2664 | 2910.7315 | 0.280 | 135.4 | 112.6 | 7147.5 | 178.7 | 0.76 |
|  |  |  |  |  | 235.3 | -8.0 | 11.7 |  |  |  |  |  | 133.9 | 117.5 | 6952.4 | 173.8 | 0.77 |
|  |  |  |  |  | 234.3 |  |  |  |  |  |  |  | 133.5 | 118.6 | 6907.2 | 172.7 | 0.77 |
|  |  | Cal-2 | S1 | 4 | 186.0 | -7.6 | 11.2 |  | 3 | 2910.3105 | 2911.4271 | 0.246 | 87.7 | 33.6 | 5812.1 | 145.3 | 0.60 |
|  |  |  |  |  | 189.0 | -7.6 | 11.2 |  |  |  |  |  | 88.6 | 36.6 | 5933.5 | 148.3 | 0.60 |
|  |  |  |  |  | 187.0 | -7.5 | 11.1 |  |  |  |  |  | 88.0 | 34.5 | 5850.1 | 146.3 | 0.60 |
|  |  |  |  |  | 187.5 | -6.0 | 9.2 |  |  |  |  |  | 88.2 | 34.9 | 5868.8 | 146.7 | 0.60 |
|  |  | Cal-2 | S2 | 4 | 185.5 | -7.6 | 11.2 |  | 4 | 2910.3105 | 2911.4271 | 0.246 | 87.6 | 33.2 | 5794.2 | 144.9 | 0.60 |
|  |  |  |  |  | 183.6 |  |  |  |  |  |  |  | 87.0 | 31.7 | 5729.4 | 143.2 | 0.61 |
|  |  |  |  |  | 184.9 | -7.8 | 11.5 |  |  |  |  |  | 87.4 | 32.7 | 5773.8 | 144.3 | 0.61 |
|  |  |  |  |  | 188.5 | -5.0 | 7.9 |  |  |  |  |  | 88.4 | 36.0 | 5911.2 | 147.8 | 0.60 |

Abbreviations: Cal-1 = first-generation calcite; Cal-2 = second-generation calcite; Qtz-1= first-generation quartz; Qtz-2 = second-generation quartz; FIAs = fluid inclusion assemblages; Th = homogenization temperature; Tm = final ice-melting temperature; νm = Raman CH4 peak shift; ν1 = real Raman CH4 peak position; ρ = methane density; Pt = trapping pressure; Plitho = (Trapping depth/1000) × 25 MPa/km, lithostatic pressure.

Table S3. Microthermometry parameters, vapor volume fraction, methane concentration, and trapping pressure of aqueous inclusions.

| Well name | Sample No. | Host mineral | FIAs No. | Aqueous inclusions (AIs) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number | Th (℃) | Tm (℃) | Salinity (wt%) | ρl (g/cm3) | Vapor volume fraction | νm | ν1 | ρb (g/cm3) | Xmeth (ppm) | Trapping pressure (MPa) |
| DY1 | DY1-M-1B | Cal-1 | F1 | 4 | 223.4 | -5.0 | 7.9 | 1.0559 | 4.0 % | 2911.4348 | 2917.3403 | 0.293 | 11410.32 | 105.7 |
|  |  |  |  |  | 218.5 | -5.2 | 8.1 | 1.0559 | 4.0% | 2911.4348 | 2917.3403 | 0.293 | 11410.32 | 122.3 |
|  |  |  |  |  | 218.5 | -5.2 | 8.1 | 1.0559 | 4.0% | 2911.4348 | 2917.3403 | 0.293 | 11410.32 | 122.3 |
|  |  |  |  |  | 218.8 | -5.2 | 8.1 | 1.0559 | 4.0% | 2911.4348 | 2917.3403 | 0.293 | 11410.32 | 121.0 |
|  |  | Cal-1 | F3 | 2 | 221.4 | -5.2 | 8.1 | 1.0559 | 4.0% | 2911.4348 | 2917.3403 | 0.293 | 11410.32 | 110.9 |
|  |  |  |  |  | 224.9 | -5.4 | 8.4 | 1.0559 | 4.0% | 2911.0872 | 2917.0990 | 0.310 | 12096.33 | 126.0 |
|  |  | Cal-2 | S1 | 3 | 202.1 | -5.0 | 7.9 | 1.0559 | 3.0% | 2910.3950 | 2916.4063 | 0.242 | 7025.96 | 73.1 |
|  |  |  |  |  | 198.8 | -4.8 | 7.6 | 1.0559 | 3.0% | 2910.3950 | 2916.4063 | 0.242 | 7025.96 | 77.9 |
|  |  |  |  |  | 199.7 | -4.8 | 7.6 | 1.0559 | 3.0% | 2910.3950 | 2916.4063 | 0.242 | 7025.96 | 66.3 |
|  |  | Cal-2 | S2 | 1 | 199.4 | -5.0 | 7.9 | 1.0559 | 3.0% | 2910.3950 | 2916.4063 | 0.242 | 7025.96 | 77.0 |
| DY3 | DY3-M-3 | Cal-1 | F1 | 4 | 223.4 | -4.9 | 7.7 | 1.0559 | 4.0% | 2911.4348 | 2917.3403 | 0.293 | 11410.32 | 106.8 |
|  |  |  |  |  | 222.8 | -5.0 | 7.9 | 1.0559 | 4.0% | 2911.4348 | 2917.3403 | 0.293 | 11410.32 | 108.1 |
|  |  |  |  |  | 218.6 | -4.9 | 7.7 | 1.0559 | 4.0% | 2911.4348 | 2917.3403 | 0.293 | 11410.32 | 121.9 |
|  |  |  |  |  | 222.4 | -5.1 | 8.0 | 1.0559 | 4.0% | 2911.4348 | 2917.3403 | 0.293 | 11410.32 | 109.1 |
|  |  | Cal-1 | F2 | 4 | 223.7 | -5.0 | 7.9 | 1.0559 | 4.0% | 2911.0872 | 2917.0990 | 0.310 | 12096.33 | 127.9 |
|  |  |  |  |  | 224.6 | -5.0 | 7.9 | 1.0559 | 4.0% | 2911.0872 | 2917.0990 | 0.310 | 12096.33 | 124.0 |
|  |  |  |  |  | 225.3 | -4.8 | 7.6 | 1.0559 | 4.0% | 2911.0872 | 2917.0990 | 0.310 | 12096.33 | 122.7 |
|  |  |  |  |  | 223.3 | -5 | 7.9 | 1.0559 | 4.0% | 2911.0872 | 2917.0990 | 0.310 | 12096.33 | 129.9 |
|  |  | Cal-2 | S1 | 4 | 198.3 | -4.8 | 7.6 | 1.0559 | 3.0% | 2912.5266 | 2918.5394 | 0.239 | 6954.93 | 75.7 |
|  |  |  |  |  | 197.9 | -4.9 | 7.7 | 1.0559 | 3.0% | 2912.5266 | 2918.5394 | 0.239 | 6954.93 | 76.6 |
|  |  |  |  |  | 195.4 | -5.0 | 7.9 | 1.0559 | 3.0% | 2912.5266 | 2918.5394 | 0.239 | 6954.93 | 80.0 |
|  |  |  |  |  | 196.1 | -5.0 | 7.9 | 1.0559 | 3.0% | 2912.5266 | 2918.5394 | 0.239 | 6954.93 | 79.1 |
|  |  | Cal-2 | S2 | 4 | 201.5 | -5.0 | 7.9 | 1.0559 | 3.0% | 2910.3950 | 2916.4063 | 0.242 | 7025.96 | 73.9 |
|  |  |  |  |  | 202.0 | -5.1 | 8.0 | 1.0559 | 3.0% | 2910.3950 | 2916.4063 | 0.242 | 7025.96 | 73.0 |
|  |  |  |  |  | 198.6 | -5.2 | 8.1 | 1.0559 | 3.0% | 2910.3950 | 2916.4063 | 0.242 | 7025.96 | 77.7 |
|  |  |  |  |  | 197.3 | -5.0 | 7.9 | 1.0559 | 3.0% | 2910.3950 | 2916.4063 | 0.242 | 7025.96 | 79.6 |
| DY4 | DY4-M-6 | Cal-1 | F1 | 4 | 238.8 | -7.6 | 11.2 | 1.0782 | 6.0% | 2912.5266 | 2918.5394 | 0.239 | 13957.78 | 132.8 |
|  |  |  |  |  | 239.0 | -7.5 | 11.1 | 1.0782 | 6.0% | 2912.5266 | 2918.5394 | 0.239 | 13957.78 | 131.9 |
|  |  |  |  |  | 239.7 | -7.6 | 11.2 | 1.0782 | 6.0% | 2912.5266 | 2918.5394 | 0.239 | 13957.78 | 125.7 |
|  |  |  |  |  | 241.5 | -7.5 | 11.1 | 1.0782 | 6.0% | 2912.5266 | 2918.5394 | 0.239 | 13957.78 | 118.2 |
|  |  | Cal-1 | F3 | 4 | 229.4 | -7.2 | 10.7 | 1.0782 | 4.0% | 2910.9792 | 2916.9909 | 0.316 | 12060.90 | 124.3 |
|  |  |  |  |  | 233.0 | -7.4 | 11.0 | 1.0782 | 5.0% | 2912.1287 | 2918.1412 | 0.258 | 12443.91 | 116.5 |
|  |  |  |  |  | 230.8 | -7.6 | 11.2 | 1.0782 | 5.0% | 2912.1287 | 2918.1412 | 0.258 | 12443.91 | 127.3 |
|  |  |  |  |  | 233.8 | -7.4 | 11.0 | 1.0782 | 5.0% | 2912.1287 | 2918.1412 | 0.258 | 12443.91 | 113.7 |
|  |  | Cal-2 | S1 | 2 | 182.4 | -7.5 | 11.1 | 1.0782 | 3.0% | 2913.8618 | 2919.8755 | 0.179 | 5096.14 | 80.0 |
|  |  |  |  |  | 187.1 | -7.4 | 11.0 | 1.0782 | 3.0% | 2913.8618 | 2919.8755 | 0.179 | 5096.14 | 74.4 |
|  |  | Cal-2 | S4 | 3 | 187.6 | -7.7 | 11.3 | 1.0782 | 3.0% | 2913.5142 | 2919.5277 | 0.194 | 5530.21 | 83.6 |
|  |  |  |  |  | 187.2 | -7.6 | 11.2 | 1.0782 | 3.0% | 2913.5142 | 2919.5277 | 0.194 | 5530.21 | 84.1 |
|  |  |  |  |  | 183.7 | -7.6 | 11.2 | 1.0782 | 3.0% | 2913.5142 | 2919.5277 | 0.194 | 5530.21 | 89.6 |
| DY5 | DY5-M-3 | Cal-1 | F1 | 3 | 237.0 | -3.8 | 6.2 | 1.0413 | 6.0% | 2912.0237 | 2918.0361 | 0.263 | 15879.71 | 117.2 |
|  |  |  |  |  | 235.6 | -4.0 | 6.4 | 1.0413 | 5.0% | 2911.3284 | 2917.3403 | 0.298 | 14833.69 | 108.8 |
|  |  |  |  |  | 233.4 | -3.8 | 6.2 | 1.0413 | 5.0% | 2911.3284 | 2917.3403 | 0.298 | 14833.69 | 117.9 |
|  |  | Cal-1 | F2 | 2 | 229.4 | -3.7 | 6.0 | 1.0413 | 5.0% | 2911.3284 | 2917.3403 | 0.298 | 14833.69 | 137.5 |
|  |  |  |  |  | 231.5 | -3.6 | 5.9 | 1.0413 | 5.0% | 2911.3284 | 2917.3403 | 0.298 | 14833.69 | 125.6 |
|  |  | Cal-2 | S1 | 3 | 185.1 | -3.8 | 6.2 | 1.0413 | 3.0% | 2914.4568 | 2920.4710 | 0.247 | 7267.50 | 85.9 |
|  |  |  |  |  | 183.6 | -3.6 | 5.9 | 1.0413 | 3.0% | 2912.7188 | 2918.7317 | 0.230 | 6787.50 | 79.6 |
|  |  |  |  |  | 183.2 | -3.6 | 5.9 | 1.0413 | 3.0% | 2912.7188 | 2918.7317 | 0.230 | 6787.50 | 80.5 |
|  |  | Cal-2 | S2 | 4 | 182.7 | -3.7 | 6.0 | 1.0413 | 3.0% | 2914.4568 | 2920.4710 | 0.247 | 7267.50 | 90.9 |
|  |  |  |  |  | 183.6 | -4.0 | 6.4 | 1.0413 | 3.0% | 2912.7188 | 2918.7317 | 0.230 | 6787.50 | 79.7 |
|  |  |  |  |  | 184.1 | -3.6 | 5.9 | 1.0413 | 3.0% | 2914.4568 | 2920.4710 | 0.247 | 7267.50 | 88.3 |
|  |  |  |  |  | 183.9 | -3.8 | 6.2 | 1.0413 | 3.0% | 2912.7188 | 2918.7317 | 0.230 | 6787.50 | 78.8 |
|  |  | Qtz-1 | F1 | 3 | 232.7 | -3.4 | 5.6 | 1.0413 | 5.0% | 2911.3284 | 2917.3403 | 0.298 | 14833.69 | 120.0 |
|  |  |  |  |  | 234.7 | -3.7 | 6.0 | 1.0413 | 6.0% | 2912.0237 | 2918.0361 | 0.263 | 15879.71 | 128.1 |
|  |  |  |  |  | 234.0 | -3.6 | 5.9 | 1.0413 | 6.0% | 2912.0237 | 2918.0361 | 0.263 | 15879.71 | 130.3 |
|  |  | Qtz-1 | F2 | 4 | 232.3 | -3.6 | 5.9 | 1.0413 | 5.0% | 2910.9792 | 2916.9909 | 0.316 | 15716.33 | 136.6 |
|  |  |  |  |  | 234.1 | -4.0 | 6.4 | 1.0413 | 5.0% | 2910.9792 | 2916.9909 | 0.316 | 15716.33 | 125.9 |
|  |  |  |  |  | 237.4 | -4.0 | 6.4 | 1.0413 | 5.0% | 2910.9792 | 2916.9909 | 0.316 | 15716.33 | 110.9 |
|  |  |  |  |  | 236.9 | -3.7 | 6.0 | 1.0413 | 5.0% | 2910.9792 | 2916.9909 | 0.316 | 15716.33 | 113.8 |
|  |  | Qtz-2 | S1 | 3 | 188.1 | -3.6 | 5.9 | 1.0413 | 4.0% | 2913.4141 | 2919.4275 | 0.198 | 7873.45 | 97.2 |
|  |  |  |  |  | 186.4 | -3.7 | 6.0 | 1.0413 | 3.0% | 2912.0237 | 2918.0361 | 0.263 | 7757.79 | 98.5 |
|  |  |  |  |  | 187.2 | -3.6 | 5.9 | 1.0413 | 4.0% | 2913.4141 | 2919.4275 | 0.198 | 7873.45 | 99.5 |
|  |  | Qtz-2 | S2 | 3 | 185.4 | -3.5 | 5.7 | 1.0413 | 3.0% | 2912.3713 | 2918.3840 | 0.247 | 7267.50 | 85.2 |
|  |  |  |  |  | 188.6 | -3.7 | 6.0 | 1.0413 | 5.0% | 2914.4568 | 2920.4710 | 0.153 | 7678.63 | 79.3 |
|  |  |  |  |  | 187.3 | -3.8 | 6.2 | 1.0413 | 3.0% | 2912.3713 | 2918.3840 | 0.247 | 7267.50 | 93.6 |
| DY2 | DY2-M-3B | Cal-1 | F1 | 3 | 236.0 | -7.2 | 10.7 | 1.0782 | 4.0% | 2910.3950 | 2916.4063 | 0.347 | 13231.46 | 127.6 |
|  |  |  |  |  | 234.2 | -7.4 | 11.0 | 1.0782 | 4.0% | 2910.3950 | 2916.4063 | 0.347 | 13231.46 | 138.2 |
|  |  |  |  |  | 238.8 | -7.2 | 10.7 | 1.0782 | 5.0% | 2911.7810 | 2917.7933 | 0.275 | 13253.71 | 115.2 |
|  |  | Cal-1 | F2 | 1 | 238.0 | -7.7 | 11.3 | 1.0782 | 5.0% | 2911.7810 | 2917.7933 | 0.275 | 13253.71 | 118.7 |
|  |  | Cal-2 | S1 | 3 | 186.0 | -7.6 | 11.2 | 1.0782 | 3.0% | 2913.5142 | 2919.5277 | 0.194 | 5530.21 | 85.6 |
|  |  |  |  |  | 189.0 | -7.6 | 11.2 | 1.0782 | 4.0% | 2914.5537 | 2920.5679 | 0.149 | 5726.10 | 85.1 |
|  |  |  |  |  | 187.0 | -7.5 | 11.1 | 1.0782 | 3.0% | 2913.5142 | 2919.5277 | 0.194 | 5530.21 | 83.7 |
|  |  | Cal-2 | S2 | 1 | 185.5 | -7.6 | 11.2 | 1.0782 | 3.0% | 2913.1682 | 2919.1815 | 0.209 | 5970.91 | 98.5 |

Abbreviations: Cal-1 = first-generation calcite; Cal-2 = second-generation calcite; Qtz-1= first-generation quartz; Qtz-2 = second-generation quartz; FIAs = fluid inclusion assemblages; Th = homogenization temperature; Tm = final ice-melting temperature; νm = Raman CH4 peak shift; ν1 = real Raman CH4 peak position; ρl = the density of the liquid phase; ρb = the density of the vapor bubble in aqueous inclusions; Xmeth = the concentration of methane in the methane-bearing aqueous inclusions.